CL249: Numerical Methods in Chemical Engineering
Assignment: Advanced Interpolation & Numerical Robustness

Focus: Lagrange Polynomials, Newton's Divided Differences, Pitfall Analysis & Method Synthesis

Assignment Objectives & Guidelines: 1. Implement numerical algorithms with a focus on edge cases, domain constraints, and input sanitization. 2. Evaluate mathematical failure modes (e.g., Runge's phenomenon, derivative instability, out-of-bounds extrapolation). 3. Synthesize multiple numerical concepts (Interpolation, Non-linear Root Finding, Linear Systems / LU Decomposition). 4. All Python solutions must be self-contained, fully commented, and include error handling. 5. Each student will solve just one problem depending on their Roll number. (Roll%3==0:Problem 1, Roll%3==1:Problem 2, else Problem 3). Use the last 3 digits of your roll number to compute the modulus.

Problem 1: The Failing Reactor — Lagrange Runge Traps & Root-Finding Synthesis

A pilot-scale exothermic reactor undergoes an irreversible liquid-phase reaction A → B. The reaction rate -rA (mol / (L·min)) has been measured experimentally at 11 non-uniformly spaced temperatures across the operating envelope [30°C, 80°C]. Your objective is to find the exact reactor temperature T* required to achieve a target reaction rate -rA,target = 4.20 mol / (L·min).

Experimental Pilot Reactor Data

| Point i | Temperature T (°C) | Measured Rate r_A (mol / (L·min)) |
| --- | --- | --- |
| 1 | 30.0 | 0.82 |
| 2 | 35.0 | 1.15 |
| 3 | 40.0 | 1.62 |
| 4 | 45.0 | 2.20 |
| 5 | 50.0 | 2.85 |
| 6 | 55.0 | 3.50 |
| 7 | 60.0 | 4.10 |
| 8 | 65.0 | 4.60 |
| 9 | 70.0 | 5.00 |
| 10 | 75.0 | 5.30 |
| 11 | 80.0 | 5.45 |

Tasks & Questions

Part A: Global Lagrange Interpolation & Runge's Phenomenon Analysis
Construct a single 10th-degree global Lagrange interpolating polynomial P10(T) using all 11 experimental data points. Evaluate P10(T) on a fine grid of 500 points across T ∈ [30, 80]°C.
• Plot the interpolating polynomial along with the experimental data points.
• Analyze the behavior near the boundaries (30°C and 80°C). Explain why severe numerical oscillations occur near the domain edges despite the polynomial passing through every data point perfectly (Runge's Phenomenon).
• Build a local 3rd-degree Newton divided-difference polynomial P3(T) using only 4  nodes (choose the nodes and give reason). Overlay this on your plot and contrast its numerical stability with P10(T).

Part B: Root-Finding Synthesis (Newton-Raphson Integration)
We define the residual equation g(T) = P(T) - 4.20 = 0 to find T*.
• Implement a Newton-Raphson solver Tk+1=Tk-g(Tk)g'  (Tk)  = T_k - g(T_k) / g'(T_k). Derive the exact derivative gg'(T) analytically for P3(T) and via high-precision central finite differences for P10(T).
• Run Newton-Raphson using P10(T) starting from initial guess T0 = 31.0°C. Document what happens. Does the algorithm converge to a physically meaningful operating temperature? Why or why not?
• Implement a bound-checking guard in your root finder to restrict iterates within [30, 80]°C. Run the safe solver for both P10(T) and P3(T) with T_0 = 58.0°C and report the final converged reactor temperature T* to 4 decimal places.

Problem 2: The Broken Pipeline — Bug Hunting & Code Reconstruction

An offshore oil terminal relies on dynamic liquid density estimates ρ(T) (kg/m³) to control crude oil flow rates through a subsea pipeline header. A junior engineer wrote a Python module intended to perform Newton's Divided-Difference interpolation on live temperature sensor data. However, during field testing, the module produced unphysical results, silent numerical errors, and runtime crashes.

Below is the faulty Python implementation currently installed on the edge server:

import numpy as np def compute_divided_diff_matrix(x, y): """ Calculates Newton divided-difference table. """ n = len(y) coef = np.zeros([n, n]) coef[:, 0] = y for j in range(1, n): for i in range(n - j): # BUGGY LINE BELOW: coef[i][j] = (coef[i+1][j-1] - coef[i][j-1]) / (x[i+j] - x[i+1]) return coef def evaluate_newton_poly(coef, x_data, x_val): n = len(x_data) - 1 p = coef[n, n] for k in range(1, n + 1): p = coef[n-k, n-k] + (x_val - x_data[n-k]) * p return p # Raw Unfiltered Pipeline Sensor Data: temp_sensor = np.array([45.0, 10.0, 25.0, 25.0, 60.0, 80.0]) # Note unsorted & duplicate 25.0! density_sensor = np.array([845.2, 870.1, 858.3, 858.3, 833.0, 815.4]) # Test Run: table = compute_divided_diff_matrix(temp_sensor, density_sensor) print("Predicted Density at 30 C:", evaluate_newton_poly(table, temp_sensor, 30.0))

Tasks & Debugging Directives

1. Bug Identification & Mathematical Correctness:
Identify and explicitly correct all four bugs in the provided script. Explain mathematically why the denominator indexing error in `compute_divided_diff_matrix` corrupts higher-order divided differences f[xi,xi+1,…,xi+k] .

2. Data Sanitization & Duplicate Handling:
Update the data preprocessing step to:
• Automatically sort the input coordinates (x, y) by strictly increasing values of x.
• Detect duplicate sensor readings (|xi-xj| < 1e-6) and remove duplicates to avoid division-by-zero errors in divided difference calculations.

3. Defensive Guard & Custom Exception:
Implement a custom exception class `ExtrapolationWarning`. Modify `evaluate_newton_poly` to check whether `x_val` falls outside [min(x), max(x)]. If out of bounds, raise `ExtrapolationWarning` and return a linear boundary extrapolation instead of the high-degree polynomial value.

4. Verification & Clean Code Output:
Run your corrected script on the raw sensor dataset. Compute and print the predicted fluid density at T = 30.0°C, T = 50.0°C, and attempt evaluation at T = 95.0°C (triggering your guard).

Sample data - # Raw sensor readings

# Format: (Temperature in °C, Density in kg/m³)

temp_sensor    = np.array([45.0, 10.0, 25.0, 25.0, 60.0, 80.0])

density_sensor = np.array([845.2, 870.1, 858.3, 858.3, 833.0, 815.4])

Use the 6 raw sensor points provided in the starter code above. Note that this dataset contains unsorted values and repeated temperature readings.  In the validation phase generate 50 evenly spaced points- eg, you can use- np.linspace(10, 80, 50) and plot the interpolated density curve. You can make the density readings as per any algorithm- eg, take a function and superpose a random component. You must also test if your code catches extreme values T=105∘, or T=-5∘ and returns the linearized boundary fallback rather than blowing up.

Problem 3: Mass Transfer Column Profiling — Method Synthesis & FLOP Complexity

In a gas-absorption packed column, volatile organic compound (VOC) concentrations C (ppm) are monitored across four non-equispaced height sampling ports z (meters). You must reconstruct the continuous spatial concentration profile C(z) using Newton's Divided Differences and couple this profile with a mass balance equation matrix system to determine inter-stage flux parameters.

Absorption Column Sampling Heights & Measured VOC Data

| Sampling Port | Height z (m) | VOC Concentration C (ppm) |
| --- | --- | --- |
| Port 1 (Bottom) | 0.0 | 250.0 |
| Port 2 | 1.2 | 110.0 |
| Port 3 | 2.8 | 42.0 |
| Port 4 (Top) | 4.0 | 15.0 |

Tasks & Synthesis Requirements

Step 1: Newton Interpolation Profile Construction
Construct the complete 3rd-degree Newton divided-difference table for C(z). Express the full interpolating polynomial C(z) in expanded nested form.

Step 2: Linear System Matrix Assembly & LU Decomposition
To calibrate four dynamic liquid-phase mass transfer parameters k = k1,k2,k3,k4T , we evaluate concentrations C(z) and concentration gradients C'=dCdz at mid-bed heights zmid ∈ {0.6, 2.0, 3.4} meters.
The governing system Ax = b is defined as:

  [ C(0.6)   C'(0.6)     0        0   ] [ k1 ]   [ 180.0 ]
  [   0      C(2.0)   C'(2.0)     0   ] [ k2 ] = [  65.0 ]
  [   0        0      C(3.4)   C'(3.4)] [ k3 ]   [  22.0 ]
  [   1        1        1        1    ] [ k4 ]   [   1.0 ]

• Evaluate C(zmid) and C'(zmid) from your Newton polynomial to populate matrix A.
• Write an LU Decomposition algorithm with partial pivoting to solve Ax = b for vector k. Report the values of k1,k2,k3,k4.

Step 3: Critical Computational Complexity Analysis (FLOP Counting)
Suppose a new real-time optical sensor is installed at height z = 2.0 m, giving a total of n+1 points.
• Compare the exact Floating-Point Operations (FLOPs) required to update the interpolation model using Lagrange Basis Polynomials versus Newton's Divided-Difference Table.
• Explain why updating the Lagrange basis requires O(n²) operations from scratch, while Newton's table allows appending a node in O(n) time. Prove this by writing a Python micro-benchmark tracking operation counts.